# Final deburr and FOD check, machined aluminium bracket

EXAMPLE for a fictional company. Every spec value is an example value, not a recommendation. Replace it with the value from your drawing, specification or qualified procedure.

From coplain.com/industries/aerospace-job-shops: how to adapt it and common mistakes are on that page.

## Document control

| Document | WI-OP40-0117, Example |
| --- | --- |
| Revision | C, matches drawing 4471-220 Rev F |
| Scope | Operation 40, deburr and FOD check after final CNC operation |
| Key characteristics | KC1 bore diameter, KC2 edge break on mounting face |
| Approved by | Quality engineer and operations lead |

## Tools, materials and PPE

- Approved hand deburr tools from the OP40 kit

- Lint-free wipes and filtered shop air

- 10× loupe

- Calibrated bore gauge, ID on the traveler

- Safety glasses

## Steps

| No. | Step | Key point | Reason |
| --- | --- | --- | --- |
| 1 | Confirm the traveler. Check that the traveler, this instruction and the drawing all show the same revision before touching the part. | Stop and call the lead if any revision differs. | Building to an old revision is a configuration error, not a workmanship one. |
| 2 | Break sharp edges. Deburr all edges to the edge-break callout on the drawing, working away from KC1. | Do not touch the bore with any tool. SAFETY: Deburr away from your hand; wear safety glasses. | Tool contact in the bore can take KC1 out of tolerance. |
| 3 | Clean and inspect for FOD. Blow out all holes, wipe the part, and inspect every cavity with the loupe. | Count the OP40 kit tools back into the shadow board before releasing the part. | A missing tool is treated as possible FOD until found. |
| 4 | Verify and record KC1. Measure the bore with the bore gauge listed on the traveler and record the value. | Record the gauge ID with the reading. | Links the measurement to a calibrated instrument. |
| 5 | Sign off. Stamp or sign the OP40 line on the traveler and move the part to the inspection rack. | Never sign for a step someone else did. | The traveler is the record of who did the work. |

## Acceptance criteria

- All edges broken per drawing; no burrs under 10× magnification

- No loose material, chips or debris in any cavity

- KC1 recorded with gauge ID and within drawing tolerance

## Sign-off

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Approved by |  | Date |  |
| Trained / signed by operator |  | Date |  |