# Setup verification, progressive die stamping

EXAMPLE for a fictional company. Every spec value is an example value, not a recommendation. Replace it with the value from your drawing, specification or qualified procedure.

From coplain.com/industries/automotive-suppliers: how to adapt it and common mistakes are on that page.

## Document control

| Document | SWI-P12-031, Example |
| --- | --- |
| Revision | D, matches control plan CP-P12 Rev H |
| Scope | Press 12, first-off verification after die change or coil change |
| Special characteristics | SC1 hole position to datum A, SC2 flange height |
| Approved by | Quality engineer, production supervisor |

## Tools, materials and PPE

- Checking fixture CF-31 with go/no-go pins

- Height gauge, ID on the setup sheet

- Setup sheet and first-off record

- Cut-resistant gloves, safety glasses, hearing protection

## Steps

| No. | Step | Key point | Reason |
| --- | --- | --- | --- |
| 1 | Confirm the setup sheet. Check the die number, coil material and setup sheet revision against the job order. | The coil tag must match the material on the control plan. | Wrong material is a common source of customer complaints. |
| 2 | Run and quarantine first parts. Run the first strip and place the first-off parts in the red first-off tray. | Do not release the press to production yet. SAFETY: Keep hands out of the die area; use the two-hand control. | Parts made before verification are not approved. |
| 3 | Check SC1 on the fixture. Load a first-off part on CF-31 and insert each go pin, then each no-go pin. | Every go pin enters and no no-go pin enters. | SC1 affects fit at the customer’s assembly. |
| 4 | Measure SC2. Measure flange height at the two marked points and record both values with the gauge ID. | Both values within the control plan limits. | Records the setup was verified with a calibrated gauge. |
| 5 | Release or react. If all checks pass, sign the first-off record and release the press. If any check fails, follow the reaction plan on the control plan and call the setup lead. | Never adjust and re-check without recording both results. | The record must show what was found and what was done. |

## Acceptance criteria

- All go pins enter and no no-go pins enter on CF-31

- SC2 values recorded and within control plan limits

- First-off record signed before production parts are packed

## Sign-off

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Approved by |  | Date |  |
| Trained / signed by operator |  | Date |  |