# Gowning for Entry to the Assembly Cleanroom

EXAMPLE for a fictional company. Every spec value is an example value, not a recommendation. Replace it with the value from your drawing, specification or qualified procedure.

From coplain.com/blog/cleanroom-gowning-work-instruction-example: how to adapt it and common mistakes are on that page.

## Document control

| Document no. | WI-CR-002 (example) |
| --- | --- |
| Revision | E |
| Applies to | Everyone entering the assembly cleanroom: operators, maintenance, visitors |
| Owner | Cleanroom supervisor |
| Approved by | Quality manager (signature on master copy) |
| Training | Gowning qualification recorded on the skills matrix before unescorted entry |

## Tools, materials and PPE

- Locker for personal items

- Hand-washing sink with soap and paper towels; hand sanitizer

- Hair cover, beard cover where needed, face mask

- Cleanroom hood, coverall and boots in the correct size

- Safety glasses or goggles

- Cleanroom gloves; 70% isopropyl alcohol spray for gloves (example value)

- Full-length mirror

## Steps

| No. | Step | Key point | Reason |
| --- | --- | --- | --- |
| 1 | Prepare before the gowning room. Remove jewelry, watches and phones and put them in your locker with outerwear. Remove makeup. Report any illness, rash or sunburn to the supervisor. | Nothing goes into the cleanroom that is not on the approved list. | Jewelry and cosmetics shed particles and can tear gloves or garments; illness raises shedding. |
| 2 | Wash your hands. Wash hands and forearms with soap for at least 20 seconds (example value), rinse, and dry with a paper towel. | Turn off the tap with the towel, not your clean hand. | Clean hands carry fewer particles and microorganisms onto the garments you are about to handle. |
| 3 | Cover hair and face. Put on the hair cover so all hair is inside, then a beard cover if you have facial hair, then the face mask. | No hair visible at the forehead, ears or neck. Mask over the nose. | Covering the head first stops hair and skin falling onto the coverall as you put it on. |
| 4 | Put on the hood. Take the hood from its pack touching only the inside. Put it on and fasten it so it covers the hair cover, ears and neck. | Handle garments by the inside wherever possible. | The outside of the garment faces the room and must stay clean. |
| 5 | Put on the coverall. Unfold the coverall without letting it touch the floor. Step in one leg at a time, pull it up, put your arms in, tuck the hood skirt inside the collar and zip up. | If any part of the coverall touches the floor, discard it and take a new one. SAFETY: Sit or hold the rail while stepping into the legs to keep your balance. | The floor on the less clean side of the gowning room is the dirtiest surface in the area. |
| 6 | Put on boots across the bench. Sit on the step-over bench. Put on one boot, tuck the coverall leg inside, fasten it and swing that leg over to the clean side. Repeat with the other leg. | A booted foot only ever touches the clean side. SAFETY: Swing your legs over slowly and keep a hand on the bench. Gowning room floors can be slippery. | The bench is the line between less clean and clean; crossing it with an unbooted foot carries dirt across. |
| 7 | Put on eye protection and gloves. Put on safety glasses or goggles. Sanitize your hands, then put on gloves, pulling each cuff over the coverall sleeve. | Glove cuffs over the sleeves, no gap at the wrist. Change any glove that tears. | Gloves go on last so they cover hands that touched everything else. |
| 8 | Sanitize gloves and check in the mirror. Spray gloves with the alcohol and let them dry. Check yourself in the full-length mirror front and back. | No skin or hair showing except around the eyes; all fasteners closed; no tears. SAFETY: Isopropyl alcohol is flammable. No ignition sources in the gowning room, and let gloves dry before touching anything electrical. | A mirror check catches gaps you cannot feel. |
| 9 | Enter and record. Enter through the airlock, letting one door close before opening the next. Sign the entry log. | Never hold both airlock doors open. | The airlock keeps the pressure difference that stops less clean air flowing in. |

## Acceptance criteria

- All garments in the correct order and size, none touched the floor or the less clean side.

- No skin or hair visible except around the eyes.

- Glove cuffs over coverall sleeves, gloves sanitized and dry.

- Mirror check done front and back with no tears or open fasteners.

- Entry recorded in the cleanroom log.

## Sign-off

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Approved by |  | Date |  |
| Trained / signed by operator |  | Date |  |