# Mold startup and first-shot approval

EXAMPLE for a fictional company. Every spec value is an example value, not a recommendation. Replace it with the value from your drawing, specification or qualified procedure.

From coplain.com/industries/plastics-injection-molding: how to adapt it and common mistakes are on that page.

## Document control

| Document | WI-MLD-022, Example |
| --- | --- |
| Revision | B, used with process sheet PS-M118-P07 Rev D |
| Scope | Startup of mold M-118 on press P-07 after a mold change |
| Critical characteristics | CC1 snap-fit width, CC2 no short shots on the latch |
| Approved by | Process engineer, quality engineer |

## Tools, materials and PPE

- Process sheet PS-M118-P07

- Dryer log for the resin lot

- Caliper, ID on the setup record

- Boundary sample board for M-118

- Heat-resistant gloves, safety glasses

## Steps

| No. | Step | Key point | Reason |
| --- | --- | --- | --- |
| 1 | Confirm material is ready. Check the dryer log shows the resin lot has dried for the time and temperature on the process sheet, and record the lot number. | Do not start with resin that has not completed its drying time. | Wet resin causes splay and weak parts that may pass a visual check. |
| 2 | Load the approved process. Load the stored setup for M-118 and compare each setting on the screen with the process sheet. | Settings outside the sheet’s window need process engineer approval. | The approved process is what the parts were qualified with. |
| 3 | Purge and run to stable. Purge the barrel, then run shots until the cushion and cycle time are steady. | Scrap the startup shots; never mix them with good parts. SAFETY: Purge into the guarded purge pan; hot material can spatter. | Parts made before the process is stable are not approved. |
| 4 | Approve the first shot. Compare one full shot with the boundary samples and measure CC1 on each cavity. | Record each cavity’s value with the caliper ID. | Every cavity must be proven, not just one. |
| 5 | Release the cell. Sign the setup record and set the press to automatic. If any check fails, stop and call the process engineer. | Do not adjust settings to chase a dimension without approval. | Unapproved adjustments move the process away from its validated window. |

## Acceptance criteria

- Material lot recorded with completed drying time

- All settings within the process sheet window

- CC1 within tolerance on every cavity; CC2 shows a full latch with no short shot

## Sign-off

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Approved by |  | Date |  |
| Trained / signed by operator |  | Date |  |