# Energy Control (Lockout/Tagout) Procedure

Free template from coplain.com/templates/lockout-tagout-procedure-template — field guide and filled Example on that page.

## Equipment and scope

| Procedure no. / revision |  |
| --- | --- |
| Machine / equipment ID |  |
| Location |  |
| Servicing tasks covered |  |
| Tasks not covered (use another procedure) |  |
| Written by / date |  |
| Approved by / date |  |

## People

| Authorized employees (trained on this procedure) |  |
| --- | --- |
| Affected employees to notify |  |
| Notification method |  |

## Energy sources

| ID | Type and magnitude | Isolation device and location | Lockout method / hardware | Stored energy release | Verification of zero energy |
| --- | --- | --- | --- | --- | --- |

## Lockout sequence

1. Prepare: identify the energy sources above and notify affected employees

2. Shut down the machine with its normal stopping procedure

3. Isolate each energy source at its isolating device

4. Apply a personal lock and tag at each isolation point (or use group lockout below)

5. Release, block or restrain stored energy

6. Verify isolation: try the start controls, check gauges, test for voltage, then return controls to off

## Release from lockout

Inspect the work area: tools removed, guards reinstalled, machine intact

Account for every person and make sure they are clear

Notify affected employees that locks are being removed

Each person removes their own lock and tag; then restore energy

## Group lockout and shift change

Group lockout method (who is in charge, lockbox use, how each person applies their lock)

Shift or personnel change handover (incoming lock on before outgoing lock off)

## Periodic inspection record (at least annually)

| Date | Inspector (not a user of this procedure) | Authorized employees observed | Deviations found | Corrective action |
| --- | --- | --- | --- | --- |