# Press brake setup and first-bend check

EXAMPLE for a fictional company. Every spec value is an example value, not a recommendation. Replace it with the value from your drawing, specification or qualified procedure.

From coplain.com/industries/metal-fabrication: how to adapt it and common mistakes are on that page.

## Document control

| Document | WI-PB-009, Example |
| --- | --- |
| Revision | C, matches drawing 2210-14 Rev B |
| Scope | Setup of press brake PB-2 for bracket 2210-14, 3 bends |
| Key dimensions | Flange A height, overall width after bend 3 |
| Approved by | Production lead, quality inspector |

## Tools, materials and PPE

- Punch and die from the tooling list on the setup sheet

- Protractor and tape measure, IDs on the setup record

- First-piece tag

- Cut-resistant gloves, safety glasses, steel-toe shoes

## Steps

| No. | Step | Key point | Reason |
| --- | --- | --- | --- |
| 1 | Lock out and change tooling. Lock out PB-2 following its LOTO procedure, then install the punch and die listed on the setup sheet. | Verify zero energy by trying the controls before you reach into the tooling area. SAFETY: Never change tooling with the ram powered. | Tooling changes put hands between the punch and die. |
| 2 | Load the program. Remove the lockout, load the program for 2210-14 and check the program revision against the setup sheet. | The program revision must match the drawing revision on the header. | An old program bends to an old drawing. |
| 3 | Bend a test blank. Bend a blank from the same material lot through all three bends in the programmed sequence. | Follow the sequence; bending out of order can make bend 3 impossible. SAFETY: Keep hands outside the light curtain; support long parts from the side. | The sequence is set so the part clears the tooling. |
| 4 | Check the first piece. Measure flange A height, each bend angle and the overall width, and record them with the gauge IDs. | Adjust the back gauge or angle correction only, then bend a new blank. | Measuring an adjusted part hides the setup error. |
| 5 | Release. Tag the approved first piece, sign the setup record and start the run. | Keep the first piece at the brake until the job is complete. | Gives operators a physical standard to compare against. |

## Acceptance criteria

- Flange A height and overall width within drawing tolerance

- All bend angles within drawing tolerance

- Setup record signed with program revision and gauge IDs

## Sign-off

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Approved by |  | Date |  |
| Trained / signed by operator |  | Date |  |